1. Introduction

Delivery of safe hospital waste management is a challenge for many countries of the world. In recent years, the generation of such wastes has increased significantly due to an increase in population, the number of healthcare facilities and the use of disposable medical products .  It is estimated that around 10%-25% of this waste is infectious and hence hazardous . In developing countries, poor sanitation practices might result in the mixing of such hazardous waste with the general waste which may exacerbate the problem of waste management . Hence a proper management of healthcare waste is vital for public health and safety. The aim of the present study is to determine the hospital waste management (HWM) practices in Pakistan which is a developing country. It is the sixth most populous country in the world and has one of the highest urbanization and population growth rates in the region , . Hospital waste in Pakistan is regulated by Hospital Waste Management Rules - 2005 (HWMR) which were notified by the Ministry of Environment in August, 2005. Periodic assessments of healthcare facilities are necessary to evaluate compliance with these rules. Unfortunately data regarding hospital waste management practices in Pakistan is scarce. Our objectives in this study are, (i) Determination of quantity and composition of hospital waste in a major city of Pakistan, (ii) Evaluation of HWM practices in selected hospitals in light of HWMR.

2. Study Area

Gujranwala is one of the largest and most densely populated cities in Pakistan. It is situated at 32.16º North and 74.18º East covering an area of 3622 Km2 at a height of 226 meters above sea level. It has a population of 4.7 million and is currently ranked as the fastest growing city in Pakistan . The city is located in the central region of the Punjab province which is also the political and economic heartland of the country. The city has seven townships and a military cantonment area. Four of the townships are predominantly urban and the remaining are rural. Table-I summarizes the inventory of hospitals in different towns of the city. The list of hospitals was obtained from the offices of Executive District Officer-Health and the population statistics were obtained from government publications . The table divides the hospitals in the four categories of public, private, trust and military hospitals. Here Trust hospitals refer to those private hospitals that are operated by charitable institutions.

Table-I Inventory of hospitals across different towns in Gujranwala

| City District | Town | Population' 000s | Hospitals (Beds) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | (Urban %age) | Public | Private | Trust | Military |
| Gujranwala | Nandipur (NPR) | 587 (66.95%) | 1 (449) | 9 (151) | 1 (250) |  |
|  | Khiali Shahpur (KPR) | 695 (56.83%) |  | 5 (120) | 2 (225) |  |
|  | Aroop (ARP) | 682 (64.66%) |  | 14 (307) | 1 (120) |  |
|  | Qila Dedar Singh (QDS) | 644 (82.45%) |  | 1 (10) | 1 (120) |  |
|  | Kamoke | 557 (35.19%) | 1 (60) | 2 (17) | 1 (20) |  |
|  | Wazirabad | 859 (30.73%) | 1 (60) | 2 (19) |  |  |
|  | Nowshehran Virkan | 549 (4.37%) | 1 (60) | 4 (37) |  |  |
|  | Military Cantt | 135 (100%) |  |  |  | 1 (400) |
| (Total) |  | 4,708 (50.53%) | 4 (629) | 37 (651) | 6 (735) | 1 (400) |

3. Methodology

The study was conducted across 12 public and private hospitals in the four predominantly urban townships of Gujranwala. These included the towns of NPR, KPR, ARP and QDS as shown in Table-I. The hospitals were categorized based on their respective location and bed size. The methodology consisted of physical segregation and weighing of the hospital wastes for 7 days each along with the determination of waste management practices using a standard questionnaire. The questionnaire was developed in accordance with HWMR. It consisted of three parts including 7 questions to gather general information, 23 questions about waste management practices and 5 questions about final disposal of the hospital waste. The questions about waste management practices gathered information about use of safety equipment, waste records, waste collection, handling & storage and waste transportation among others. The questionnaire was addressed to the hospital manager and it was usually filled by a member of the hospital administration/waste management team. The responses were then compared with actual findings following a 7-day inspection of each hospital.

4. Results and Discussion

4.1 Waste Generation

The average generation rate for the general waste was found to be 0.497 Kg/bed day-1 while that for infectious waste was found to be 0.1702 Kg/bed day-1. Thus the average total waste generation rate was 0.667 Kg/bed day-1. The results are similar to those in other findings where it was 0.61 Kg/bed day-1 in Gansu, China , 0.62 Kg/bed day-1 in Lao PDR , 0.74 Kg/bed day-1 in Shandong, China  and 0.63 Kg/bed day-1 in Istanbul, Turkey . Table-II displays the waste generated at each hospital. The hospital names have been replaced with abbreviations such as G for government, T for trust and P for private hospitals. The table also displays the average number of in-patients (Occupancy) and outdoor patients (OPD) apart from the paramedic and sanitary staff at each hospital.

Table-II Average waste generation across surveyed hospitals

| Town | Hospital | Category | Beds | Occupancy |  | Staff |  | OPD | Waste (Kg/day) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | No. | %age | Paramedic | Sanitary |  | General | Infectious |
| NPR | G | General | 449 | 287 | 63.92% | 270 | 87 | 2882 | 152.48 | 71.21 |
| NPR | T1 | General | 250 | 100 | 40.00% | 89 | 33 | 425 | 168.34 | 6.64 |
| NPR | P2 | General | 40 | 19 | 47.50% | 21 | 9 | 200 | 37.67 | 3.14 |
| NPR | P4 | General | 35 | 26 | 74.29% | 46 | 10 | 150 | 53.94 | 10.71 |
| NPR | P6 | ENT | 11 | 2 | 18.18% | 22 | 3 | 11 | 11.26 | 5.97 |
| KPR | T2 | General | 150 | 44 | 29.33% | 87 | 19 | 515 | 40.11 | 40.25 |
| KPR | T5 | General | 75 | 42 | 56.00% | 58 | 26 | 321 | 54.27 | 40.25 |
| ARP | T3 | General | 120 | 14 | 11.67% | 33 | 6 | 101 | 32.25 | 4.74 |
| ARP | P1 | Orthopedic | 55 | 12 | 21.82% | 26 | 14 | 90 | 43.53 | 2.47 |
| ARP | P3 | Pediatrics | 37 | 20 | 54.05% | 40 | 4 | 87 | 44.10 | 16.42 |
| ARP | P5 | Orthopedic | 22 | 10 | 45.45% | 13 | 8 | 42 | 11.21 | 11.44 |
| QDS | T4 | General | 120 | 18 | 15.00% | 70 | 17 | 575 | 35.77 | 13.09 |

Figure-I highlights the change in hospital waste generation rate with the variables of hospital size (no. of beds), location, category(general vs. specialized) and type (public vs. trust vs. private). It can be seen that on average, waste generation was greatest in small hospitals (beds<50) followed by medium sized hospitals (50<beds<100) and large hospitals (beds>100). Moreover hospitals in NPR town generated the greatest amount of waste on average while those in KPR town generated the largest amount of infectious waste. Similarly waste generation was greatest in private hospitals followed by the trust hospitals and the public hospital, in that order. Specialized hospitals generated more waste on average than general hospitals.

Figure-I Variation in waste generation with hospital location, type, category and size.

Of the total hospital waste, 73.85% consisted of general, 25.8% consisted of hazardous infectious and 0.87% consisted of sharps waste. Hazardous infectious waste usually comprised of empty plastic drips, used blood infusion bags, cotton swabs, dressings and plasters, syringes, used testing kits, laboratory sample containers, NG Tubes, etc. Sharps waste consisted of all sharp objects including needles, broken vials, cut glass, etc. The general waste consisted of all waste meant to be thrown in the municipal container. On average, the percentage compositions of paper, plastic, textiles, glass, rubber, metals and others in general waste stood at 15.76%, 13.41%, 21.77%, 6.47%, 1.99%, 0.44% and 40.17% respectively. Here 'others' refer to non-risk items such as dirt, food waste, fruit peels, etc.

The results shown in Table-II were used to generate a regression based model of hospital waste generation using SPSS v.21. The independent variables included average occupancy, number of beds, no. of that out-patients and no. of sanitary staff. Table-III highlights  the results of linear regression. It can be seen that the occupancy and no. of beds were the only significant (p<0.05) variables that could be used to predict waste generation at the hospitals. The hospital waste  was found to decrease with the no. of beds and increase with the average occupancy at a hospital. The results are consistent with findings in other studies where clinical waste was found to decrease with an increase in the total number of beds at a hospital  and increase with the occupied beds .

Table-III Linear regression results

| Variable | B | T | Significance of t |
| --- | --- | --- | --- |
| Occupancy | .021 | 3.514 | .01 |
| Beds | -.009 | -4.7 | .002 |
| Out Patients | -.001 | -1.468 | .186 |
| Sanitary Staff | -.012 | -0.639 | .543 |
| Constant | 1.507 | 9.717 | .000 |
| R2 | 0.823 |  |  |
| Adjusted R2 | 0.721 |  |  |
| Sum of Squares | 2.399 |  |  |
| Residual Sum of Squares | 0.517 |  |  |
| F | 8.117 |  |  |
| Significance of F | 0.009 |  |  |

4.2 Waste Segregation

Waste segregation at most of the hospitals was either absent or incomplete. Despite the provision of color coding in HWMR the hospitals were not following proper labeling and coding of hospital waste. Consequently at T1, T2, P1, P2 and P5 hospitals waste was not being segregated at source. At T1 hospital only empty drip sets were being segregated for treatment. Similarly, At P6 hospital, all the waste was being dumped into the municipal containers without any segregation. Additionally at P2, P3 and P5 hospitals sharps were not being segregated. At P2 hospital the waste segregated as general waste was found to consist of human tissues. Similarly diapers containing patient urine and excretions were being thrown in municipal drums at all the hospitals. Poor segregation of healthcare wastes has also been noted elsewhere such as in Nigeria ,  Serbia  and Iran .

4.3 Waste Storage and Transportation

HWMR specify the internal transportation of hazardous waste using a covered waste collection trolley. The rules also call for a secure central storage facility dedicated for storing infectious waste and mandate the use of safety equipment while handling hospital waste. Off-site transportation of hospital waste is supposed to be the responsibility of the local municipal committee. In our study we found that apart from T4, sanitary workers in all the surveyed hospitals did not have the necessary safety equipment for waste collection and handling. Internal transportation of the waste at all hospitals was carried out using uncovered two-wheeled carts or by carrying waste bags in hand. All hospitals except P6 had a store room for the storage of hazardous infectious waste and sharps. Offsite transportation of general waste at T2, T4, T5, P2, P3 and P4 was carried out by scavengers to the nearby municipal containers using donkey carts. At T1 the general waste was carried away by a hospital owned vehicle to a nearby municipal container. At  P5 and P6, offsite transportation of the general waste was carried out by the hospital sanitary staff by hand. At all the hospitals, except P6, offsite transportation of hazardous infectious waste and sharps and sharps was carried out on a biweekly basis. During the meantime the waste remained in the storage area without the provision of refrigeration, in violation of HWMR. Offsite transportation of this waste was carried out by a commercial firm contracted to incinerate this waste.  Transportation was carried out in a special vehicle which was labeled with warning signs. The transportation was carried out without any optimal routing system. Such a mechanism can be established using geographic information system .

4.4 Waste Disposal & Resale

HWMR recommend the treatment of hazardous infectious waste and sharps by land-filling, incineration or any other method permitted by the law. In case of incineration, the residual ash is required to be filled in land such that it does not intoxicate ground water.  Our survey revealed that except P6, the hazardous infectious waste and sharp boxes  of all the hospitals were treated by incineration. At P6, the waste was thrown directly into the municipal containers without any pre-treatment. At the time of the survey, there was only one incineration plant in the city which was situated at the Combined Military Hospital (CMH) located in the Military Cantonment area. However it was found to be non-operational due to a lack of technical staff for its operation and maintenance. It was discovered that a commercial firm had been contracted by the surveyed hospitals for incinerating the hospital wastes. The incineration plant was located around 70 Km away in another city. Transportation of the hospital waste at such a long distance represents significant costs which are indirectly borne by the patients. Hence there exists an urgent need to find an alternate solution to this challenge.

The disposal of general waste from the hospitals took place in an open dumping site at the outskirts of the city. Here scavengers extracted different recyclable items from the waste which eventually reached a wholesale market in the nearby town of Muridke. The prices of different articles sold here included 0.07 to 0.08 US $/ Kg for paper, 0.3 $/Kg for plastic, 0.08 $/Kg for glass and 0.35-0.4 $/Kg for metal as per the exchange rate between Pakistani Rupees  and US dollars at the time of the survey.

5. Conclusions

The generation of hospital waste in Gujranwala city was found to be 912.8 Kg/day at the surveyed hospitals with a weighted average of 0.667 Kg/bed day-1. Average waste generation was found to be relatively greater in smaller private hospitals. Specialized hospitals generated more waste than general hospitals. Regression analysis showed that waste generation increased with occupancy rate but decreased with no. of beds. It was also found that the waste segregation, storage and transportation services at most of the surveyed hospitals did not meet the requirements of HWMR. Moreover, infectious waste items found their way in general waste which was sorted by scavengers for onward sale. Hence a stricter enforcement of HWMR is required at the hospitals along with necessary support from the government.

6. Acknowledgments

This work was funded by The China Specialized Research Fund for Doctoral Program of Higher Education [20120092110039]; The National Natural Science Foundation of China [71172044 and 71273047]; The National Social Science Foundation of China [12&ZD20].

7. References

Arab, M., Rouhollah Askari, B., Tajvar, M., Pourreza, A., Tajvar, M., Omrani, G., Mahmoudi, M., 2008. Report: The assessment of hospital waste management: a case study in Tehran. Waste Management & Research 26, 304-308.

Bank, T.W. (2010-2014a). Population growth (annual %).  Retrieved 23 August 2015

from The World Bank http://data.worldbank.org/indicator/SP.POP.GROW

Bank, T.W. (2010-2014b). Urban population (% of total).  Retrieved 23 August 2015

from The World Bank http://data.worldbank.org/indicator/SP.URB.TOTL.IN.ZS

Bazrafshan, E., Mostafapoor, F.K., 2011. Survey of medical waste characterization and management in Iran: a case study of Sistan and Baluchestan Province. Waste Manag Res 29, 442-450.

Birpinar, M.E., Bilgili, M.S., Erdogan, T., 2009. Medical waste management in Turkey: A case study of Istanbul. Waste Manag 29, 445-448.

Bureau of Statistics, G.o.t.P., Pakistan. (2014). Punjab Development Statistics.   http://www.bos.gop.pk/publicationreports

Gai, R., Kuroiwa, C., Xu, L., Wang, X., Zhang, Y., Li, H., Zhou, C., He, J., Tang, W., 2009. Hospital medical waste management in Shandong Province, China. Waste Manag Res 27, 336-342.

Idowu, I., Alo, B., Atherton, W., Al Khaddar, R., 2013. Profile of medical waste management in two healthcare facilities in Lagos, Nigeria: a case study. Waste Manag Res 31, 494-501.

Mayors, C. (2011). The world’s fastest growing cities and urban areas from 2006 to 2020.   http://www.citymayors.com/statistics/urban_growth1.html

Mohee, R., 2005. Medical wastes characterisation in healthcare institutions in Mauritius. Waste Management 25, 575-581.

Patwary, M.A., O'Hare, W.T., Street, G., Maudood Elahi, K., Hossain, S.S., Sarker, M.H., 2009. Quantitative assessment of medical waste generation in the capital city of Bangladesh. Waste Manag 29, 2392-2397.

Phengxay, S., 2005. Health-care waste management in Lao PDR: a case study. Waste Management & Research 23, 571-581.

Shanmugasundaram, J., Soulalay, V., Chettiyappan, V., 2012. Geographic information system-based healthcare waste management planning for treatment site location and optimal transportation routeing. Waste Manag Res 30, 587-595.

Stankovic, A., Nikic, D., Nikolic, M., 2008. Report: Treatment of medical waste in Nisava and Toplica districts, Serbia. Waste Manag Res 26, 309-313.

Taghipour, H., Mosaferi, M., 2009. The challenge of medical waste management: a case study in northwest Iran-Tabriz. Waste Manag Res 27, 328-335.

Thakur, V., Ramesh, A., 2015. Healthcare waste management research: A structured analysis and review (2005-2014). Waste Manag Res.

Yves Chartier, J.E., Ute Pieper, Annette Prüss, Philip Rushbrook, Ruth Stringer, William Townend, Susan Wilburn and Raki Zghondi, 2013. Safe Management of Wastes from Health Care Activities (second ed.) World Health Organization, Geneva

Zhang, H.J., Zhang, Y.H., Wang, Y., Yang, Y.H., Zhang, J., Wang, Y.L., Wang, J.L., 2013. Investigation of medical waste management in Gansu Province, China. Waste Manag Res 31, 655-659.